Roll No. ……………………………………………………………

NEELKANTH INSTITUTE OF TECHNOLOGY

B.Tech ECE (Semester V)

FIRST SESSIONAL EXAMINATION 2015-2016

PRINCIPLE OF COMMUNICATION (NEC-502)

Time:  1:30 Hours                                                     Total Marks 30

NOTE: - i. be precise in your Answer

ii. All section are compulsory

SECTION A

- Attempt all the Questions:                                  1X10=10

- A periodic signal x(t) is fed as input to a filter having impulse response h(t). The output of the filter would be

- Not necessarily periodic

- Necessarily periodic, with same period of factor there.

- Generally not periodic, but if so then having the same period.

- Necessarily periodic but having a longer period.

- The following is not an advantage of SSB over DSB-Full carrier AM.

- Transmitter circuit are more stable, giving better reception.

- The signal is more immune to noise.

- Much less power is required for the same signal strength.

- More channel space becomes available.

- Power saving in AM-DSB/SC is________________.

- Bandwidth required to transmit AM SSB/SC is ____.

- Draw the Ring modulator diode circuit.

- In the envelope detection diagonal clipping occurs due to __________high/low modulating frequency.

- Vestigial sideband transmission comprises between _____________.

- The basic advantage of VSB is_______________.

- In the FM if the modulation index increases the number of sideband ________ increase/decrease.

- Frequency deviation controls the______ BW/mf.

SECTION B

- Attempt any Five Question :                                 2X5=10

- What is the relationship between FM and PM & Write the Carson’s rule.

- Define the Modulation index in AM, FM & PM.

- Derive an expression for power contained in AM-DSB/FC signal.

- Classify the analog modulation techniques.

- Define modulation and why it is required?

- The transmitter radiates the 1200 W of power under carrier is modulated by two tone of 20% and 40% respectively. Determine the total power radiated.

- In an aerial Current (RMS) before modulation is 10A. After modulating it to 11.6A. Determine the modulation Pmod. If the carrier power is 10kW, what is the power after modulation?

SECTION C

- Attempt any Two  Questions:                                5X2=10

- Explain the VSB transmission concept, its advantage, recovery of modulating signal from the VSB signal, the basic feature of VSB transmission.

- The input to an envelope detector is single tone AM Signal xAM(t)=A(1+ma cos (wm t))*cos(wc t), where ma is constant, 0< ma<1 and wc>> wm.

Show if the detector output is to follow the envelope of xAM(t) it is required that at any time t0

1RC(time constant)≥wm(masinwmt01+macoswmt0)

OR

Briefly explain the square law demodulator (nonlinear) AM DSB/SC.

- Explain the generation and detection of AM-SSB/SC.